Lab #2

Pully

Class: ENGR 3350-001

Lab Section: 202

Time/Day: TR 2:00-3:15

Authors: Hannah Bishop and Justis Findley

Team:7

Start Date: September 29th, 2025

End Date: November 1st, 2025

Abstract:

The overall purpose of this lab was to 3D print a pulley assembly made from ABS plastic. The assembly had three main parts: the U-shaped assembly, the pulley wheel, and the wheel fixture. This assembly would be tested to discover the best weight-to-weight ratio it could withstand. This lab’s main goal was for us students to be able to practice additive manufacturing and to work on our own design to see how the design could change through the different groups.

Introduction:

The objective of this lab was to create a three-part assembly that was capable of meeting the different requirements necessary to complete the experiment. The assembled parts are highly recommended to be within the given ranges to ensure maximum efficiency. The given range was the material used, ABS plastic, and the plate size we could 3-D print on. The pulley should have allowed the wheel holding the rope to move freely. The assembly was tested through weights at the end of a rope, and by fixing one of the ends, which would exhibit the assembly's ability. We did some in-the-field tests, but most of the testing used a stress analysis to identify where the stress was the strongest and where the material was most likely to fail first.

Theory:

The initially designed pully offers a baseline where one can see the weak points when using a FEA (Finite Element Analysis). From the results of the FEA a new verson can be produced which in theory will be stronger and lighter than the first. This process will be repeated again until it can't be better or until time has run out.

Equipment:

- Inventor Software

- Fuson 360

- Bamboo lab studio

- Bamboo lab X-1 Carbon

- Crain Scale

- Rope

- Slow-motion Camera

- Drill Bit

- Sand paper

Procedure:

Design:

- Inventor software opening a metric sheet to begin the sketches.

- The x-axis was used to start the 2-D sketch when designing the U-assembly. The basic  shape was made on the X-plane and made to be 100mm in length and 133mm in height.

- A 34mm wide and 100mm tall gap between the walls.  After finishing the 2-D shape, we moved to the 3-D space using the extrude and added 100mm in length outwards.

- On top of the U-shape, a second 2-D sketch was started. A circle with a 25mm diameter was added to fix the U shape. That part was extruded 25mm.

- For the last 2D sketch, we put a 25mm diameter circle. In the 3-D space, the circle was cut through both sides of the object.

- A pin was made by starting a 2D sketch of 24.75 mm and extruding it 125mm.

- A new plane vertical to the extruded circle.

- A 3mm hole was added and cut all the way through the material to put a small pin through.

- New plane at the opposite end, a 35mm circle was added and extended 5mm.

- The wheel started with a 60mm diameter circle and was then extruded 32mm.

- A circle for the pin was added, putting a 25.25mm circle and cutting fully through the object.

- Adding a new axis halfway through the circle, a 45mm circle was added and cut on both sides for a total of a 10mm notch.

- The pulley assembly was made by placing the U-assembly and constraining the pin to the holes on the U-assembly. By pressing the constrain bottom then choosing mate I used the 35mm circle to the flat wall of the U-assembly it had an offset of 34mm.

- A second constrain condition  was needed to center the circle. Mate was chosen again then aligned was chosen. Clicking the center of the pin and the inside of the circle on the U-assembly.

- Then, the wheel was constrained to the pin by the middle hole being mated with the center of the pin.

Printing:

- The printer was set up to print with PLA.

- The model was set up to print with 2 walls, 3 bottom layers, and 4 top layers with a rectilinear pattern. It also uses 20% gyroid infill.

- Note all prints will use auto tree support with a threshold angle of 30 degrees.

- The next printed version “V4” had 10 walls, 5 bottom layers, and 5 top layers with rectilinear pattern. It aloes uses 25% gyroid infill.

- The next printed version “V5” had 6 walls, 6 bottom layers, and 6 top layers with concentric pattern. It aloes uses 20% gyroid infill.

- All versions starting now used 0.6 mm nozzle.

- The next printed version “V6” had 5 walls, 4 bottom layers, and 3 top layers with Hilbert Curve pattern. It aloes uses 20% gyroid infill.

- The next printed version “V7” had 5 walls, 4 bottom layers, and 3 top layers with concentric pattern. It aloes uses 20% gyroid infill.

- The next printed version “Housing V7.5” had 6 walls, 6 bottom layers, and 6 top layers with concentric pattern. It aloes uses 15% gyroid infill.

- The Pully for “Housing V7.5” had 3 walls, 3 bottom layers, and 3 top layers with concentric pattern.

- The Pin for “Housing V7.5” had 4 walls, 4 bottom layers, and 4 top layers with concentric pattern. With a 20% gyroid infill.

- The next printed version “Housing V8” had 6 walls, 6 bottom layers, and 6 top layers with concentric pattern. It aloes uses 15% gyroid infill.

- The Pully for “Housing V8” had 3 walls, 3 bottom layers, and 3 top layers with concentric pattern.

- The Pin for “Housing V8” had 4 walls, 4 bottom layers, and 4 top layers with concentric pattern. With a 20% gyroid infill.

- New pin V3 were tested with settings 2 walls, 2 bottom layers, and 2 top layers with concentric pattern. One had 5% than 10% and 15% gyroid infill.

- New pully V4 were tested with settings 2 walls, 2 bottom layers, and 2 top layers with concentric pattern. One had 5% and 10% gyroid infill.

- The next printed version “V10” had 6 walls, 6 bottom layers, and 6 top layers with concentric pattern. It aloes uses 15% gyroid infill.

- V10 was also tested at 10 walls, 10 bottom layers, and 10 top layers with concentric pattern.

- Both V10 versions were also scaled down to 75% and 50%.

- All V10 prints had tolerance issues when printed with PHAT-CF so new version “V10-CF” only had adjusted tolerances.

- The next printed version “V10-CF” at 60% scale had 8 walls, 8 bottom layers, and 8 top layers with concentric pattern. It aloes uses 10% gyroid infill.

- The Pully for “V10-CF” had 4 walls, 4 bottom layers, and 4 top layers with concentric pattern and 15% gyroid infill.

Data:

- The first modified version with a print time of 3 hours and 35 minutes weighed 22.7 grams and only healed ~50 LB and broke at the pin support for a ratio of 2.2.

- V4 with a print time of 7 hours and 47 minutes weighed 78.6 grams and healed ~606 LB and broke at the top hole for a ratio of 7.7.

- V5 with a print time of 6 hours and 23 minutes weighed 63.4 grams and healed ~608 LB and broke at the top hole for a ratio of 9.59.

- V6 with a print time of 5 hours and 45 minutes weighed 51.9 grams and healed ~397 LB and broke at the pin support for a ratio of 7.65.

- V7 with a print time of 4 hours and 41 minutes weighed 48.5 grams and healed ~382 LB and broke at the pin support for a ratio of 7.88.

- V7.5 with a print time of 1 hours and 40 minutes weighed 57.3 grams and healed ~664 LB and broke at the pin for a ratio of 11.59.

- V8 with a print time of 1 hours and 43 minutes weighed 57.6 grams and healed ~672 LB and broke at the pully for a ratio of 11.67.

- V10 weighed 56.4 grams and healed ~378 LB and broke at the pin support for a ratio of 6.7.

- V10-CF weighed 52.6 grams and healed ~794 LB and broke at the pin support for a ratio of 15.1.

- V10 (75%) weighed 28.8 grams and healed ~350 LB and did not break for a ratio of 12.15.

- V10 (50%) weighed 9.9 grams and healed ~249 LB and broke at the pin support for a ratio of 25.15.

- V10-CF (50%) weighed 10.6 grams and healed ~332 LB and broke at the pin support for a ratio of 31.32.

- V10-CF (50%) weighed 9.71 grams and healed ~390 LB and broke at the top hole for a ratio of 40.16.

Discussion and Analysis:

Understanding the software being used and how the equipment being used works is extreamly important when optimizing. Although we did not change print orientation, it is important due to the way additive manufacturing bonds materials. Thus the weak points are usually between the layer lines. Although model design is important the print settings have lots of effects. As an example, V7 and V7.5 were the same model however V7 had a weight of 48.5 grams and a ratio of 7.88 whereas V7.5 weighed 57.3 grams and had a ratio of 11.59. In the design process it is important to understand that sharp corners create stress constriction. In the real world some parts are designed to be weaker so when frailer happens it will be cheaper and quicker to fix than replacing the whole assembly. Our final pully has not been tested due to uncontrollable circumstances with the lab. Testing methods changed several times so results may vary.

Conclusions:

This lab was crucial to the development of engineering students. We were introduced to new machines and skills vital for any engineer in the workplace. We practiced using new software and machines that can help us learn about additive manufacturing. Measurements. We saw firsthand the importance of thinking through a process, where the design would be the weakest, and the best way to fix those weaknesses. It introduced us to new knowledge that will help us form a solid foundation to continue to grow as engineers.

V2

V4

V5

V6

V7

V7.5 and 8

V10 “75% & 50%”

V10-CF (60%)

Pin

U-Assembly

Wheel

Finished Pully.